Research Theme 4 – Intelligent Remote Sensing of the Near-Earth Space Environment

The rate TREx will produce data will be so large that if it is all kept, data management will be too costly. It is likely that because of clouds, lack of activity, etc., the full resolution data will only need to be collected 10% of the time or less. Via a partnership with IBM Canada, TREx will include a stream computing and sensor web component enabling autonomous “on the fly” decisions, increasing or decreasing the rate data is produced, in response to the conditions being measured by the network. With TREx, Canadian researchers will lead ground-based space science into era of data streams, and be positioned as significant players in the global initiative to turn the planet into a sensor web.

A sensor web is a dynamic, adaptable, and autonomous network of sensors. Using Artificial Intelligence (AI), a sensor web reacts in Real-Time (RT) to information from its instruments, invoking information sharing, and autonomous decision making, feedback, and control to provide a combination of greatly enhanced information content in the overall data and/or vastly improved resource efficiency. Research into sensor webs is rapidly expanding, and they are emerging as the only viable path forward for large-scale (Big Data) Earth observing systems.

TREx will be a heterogeneous network of high data volume instruments operated as a sensor web. It will use adaptive control systems built on stream computing infrastructure (InfoSphere Streams and DB2, together referred to herein as “Stream Computing”) provided through partnership with IBM. This will address challenges inherent to remote sensor operation, and enable high quality observations heretofore impossible to sustain for projects of this relatively small scope. TREx development will couple to active research areas in machine learning and space weather, and directly benefit Canadian industry by providing Artificial Intelligence (AI) solutions for high data volume, remote operation applications.

Figure 4: Sensor Web Components: 1) acquisition of data by sensors; 2) RT analytics on data streams from the sensors; 3) sharing derived information; 4) use of that information for feedback and control of the sensor web.

A sensor web has four key components. UofC Space Physics has developed the RT Environment Monitoring Platform (http://rtemp.ca), which will serve as the communications backbone of the TREx sensor web. This currently passive system receives asynchronous data packets from remote sensors, and categorises and displays data based on specified protocols. At present, RTEMP handles ~80,000 RT data streams from >100 instruments and support infrastructure at >50 sites across Canada and around the world. Donovan’s team uses RTEMP for manual assessment of field conditions and instrument health in aid of operating the array. RTEMP provides a framework for a sensor web, but at present incorporates no inline analytics, and provides no autonomous feedback and control. Sensor data acquisition and enabling sensors to receive feedback (1&4, from Figure 4) will be requirements for sensor design (UofC, Spectral Applied Research, NovAtel, and Siena College). The challenge here is to implement RT analytics and intraweb communication (2&3, Figure 4).

Challenge: TREx instrumentation must react in RT to what it is measuring. In other words, make TREx a sensor web. Inline analytics of streaming data will extract information content, for example determining viewing conditions, auroral type, and character of auroral activity. Sometimes, this will be straightforward, such as using stars in ASI images to determine a transition from clear to cloudy skies, which would trigger a dramatic decrease of imaging cadence. In other instances the information is more difficult to extract, such as the type of aurora and its temporal evolution which might trigger a “burst” mode. There are numerous techniques available to TREx for extracting this type of information from data streams, with one example being using machine vision to classify auroral type (e.g., Syrjäsuo and Donovan, Annales. Geophys., 2004).

Feature extraction and image classification analytics will be carried out by Stream Computing Nodes at each site. The outputs will be low bandwidth streams (time series) of derived quantities conveying conditions as measured by the instruments. Sometimes this information will trigger autonomous action at that site, such as a change in imaging cadence in response to viewing conditions. These streams will also be piped through RTEMP to a central Stream Computing node (the “Array Control Node” in Infrastructure). There, inline analytics carried out on all the streams will decide the optimal mode of operation of the array, which will be streamed back to the remote nodes via RTEMP. The remote nodes will then issue commands to the instruments. This blend of central control achieved through autonomous analysis of the output of remote inline analytics will enable a true sensor web in spite of communication links between nodes that are low-bandwidth and unreliable.

Challenge: Enabling the central node in the sensor web to make decisions. The sensors, stream computing software, feature extraction and image classification algorithms are the architecture of the sensor web. For this to work, the analytics at the central node must make autonomous decisions based on the information it receives via dozens to perhaps 100 streams from the 10 TREx remote sites. To do this, the team will develop models Thus, this represents a high-dimensional  is a high-dimension These decisio uses several parameters such as aurora type, weather information, and activity at neighbouring sites to predict the probability and type of aurora happening at a remote site. Based on this model and the data available at each moment of time through the whole network, the frame rate, the types of the pictures and their resolutions are determined and transmitted back to the remote sensors. The AI system will also have a feedback loop where the model is continuously improved based on the feedback from the sensors. For example, if the frame rate at one remote node is predicted incorrectly, the model will be modified to obtain better results in the future.